Estimating Water Consumption in Camp Settlings

You visit a camp with 3300 residents and 4 wells which are equipped with handpumps.  They claim that there is not enough water and there is a queue at the wells for 13 hours per day.  You visit each well and monitor the production at each by timing the filling of a few containers typically over a 15 minute period.  The data are presented below:

What is the water consumption (collection rate) in this camp?   Assume all of the water collectors are from the camp population.

| Well 1 Time L. removed | Well 2 Time L. removed | Well 3 Time L. removed | Well 4 Time L. removed |
| --- | --- | --- | --- |
| 10:07 20 | 10:58 10 | 12:42 20 | 1:22 20 |
| 10:09 20 | 11:01 20 | 12:44 20 | 1:29 20 |
| 10:10 10 | 11:05 20 | 12:46 20 | 1:35 20 |
| 10:14 20 | 11:10 20 | 12:49 20 | 1:42 20 |
| 10:17 20 | 11:13 20 | 12:51 20 |  |
| 10:19 20 |  | 12:54 20 |  |
| 10:24 20 (fight) |  |  |  |